L30 STAIRS/WALKWAYS/BALUSTRADES

BALUSTRADE(S) AND HANDRAILS(S)

Preliminary Remarks

These NBS specifications are intended to simplify the specification process but should be read in conjunction with current building regulations and British standards as well as manufacturers test certificates to ensure the system has full compliance.

We also recommend that you visit our website for CAD drawings, safety certificates, portfolio, news and essential contact details.

Our website address: 	www.q-railing.com

Manufacturer:	Q-railing

Product name:  Duo Line, is a flexible clamp baluster railing system with Anthracite grey powder-coated aluminium balusters.

Technical description, Quality characteristics:

-      Twin post Aluminium system measuring 60mm x 38mm formed in 12mm flat posts powder coated to RAL 7016.

-      Maintenance friendly does not readily corrode, rust or stain.

-      Horizontal uniformly distributed line load, uniformly distributed load applied to infill, and point load to infill testing is available

-      Various base fitting options

Usage:

-      External use, Aluminium twin post system powder coated to RAL 7016.

-      Interchangeable stainless steel modular components for various infill’s of either; - bar, web, glass clamps, glass holders, glass adapters or glass frame tube.

Quality/Safety

Even top quality materials are only as good as the workmanship used to install them. Easy installation techniques are created by modular parts with maximum precision fittings. Safety is very important in railing manufacture, vertical and horizontal loadings must be adsorbed. Reference should be made between product test certification and loading requirements set out in the current building regulations and British standards to ensure that the overall system has full compliance.

Railing profile: 60 x 12mm flat twin posts forming overall post size of 60 x 38mm

Material:

Aluminium Powder coated – Anthracite Grey – RAL 7016

Baluster Posts

-       60 x 12mm solid aluminium flat twin posts forming overall post size of 60 x 38mm. Powder coated – Anthracite Grey – RAL 7016

-       16954506033 – Height = 1000mm for top mount solutions

-        16954606033 – Height = 1300mm for fascia mounted applications

Floor flanges/railing post mounts

-     10902306033 - Floor flanges for top mounting application measuring 102mm x 142mm with 4 No. 14mm dia. Holes for bolting down to substrate.  Cover cap 10.9522.060.33 measuring 110mm x 150mm square.

-     10902506033 – Baluster bracket option for side mounting of the posts with 2 No. 14mm diameter holes for fixing to the substrate.

-     10952306033 - Baluster cover cap for covering posts when the balusters are mounted in the floor structure

5.4.1) Handrail fixing methods:

Handrail to be connected to the top of the posts using MOD 9004 for top mounting of handrail fixings. The fixing has a M10 thread to fix:-

-       MOD 0811 for mounting of horizontal cross rails.

-       MOD 0812 for mounting of raking cross rails.

-       MOD 1800, 1801, 1802 for mounting of variable handrails to accommodate flat rails, 33.7, 42.4, 48.3 diameter tubular handrail.  Fixing plates also vary in angle for changes in direction from straight, 90 degrees, 135 degrees angles.

5.4.2) Handrail options

All handrail options are available for use with this system.

-        MOD 8925/0900 for either 33.7, 38, 42.4, 48.3, 60.3 diameter

-        MOD 4900 for 60 x 30 x 2.6mm rectangular handrail

-        MOD 4925/4900 for 40 x 40 x 2.0mm square handrail

5.5 Infill:

Various infill’s are available and are to be fixed to the posts using MOD 9002 fixing brackets, the fixings vary in accordance with the clamping system from M6, M8.

-          MOD 0860 fixing brackets available in 304/316 grade to fix MOD 6920/6925 30mm diameter glass frame tube. Rubber gaskets MOD 50 are then to be inserted to accommodate glass infill varying in thickness from 8 to 10.76mm. Also available are modular connections MOD 6790 for straight connections, MOD 6313 for 90 degrees corners, MOD 6302 for 22-55˚ downwards, MOD 6312 for 22-55˚ Upwards connections. Glass tube end caps are MOD 6732.

-           MOD 52 glass clamps, to accommodate glass varying in thickness from 8 to 12.76mm glass.

-           MOD 322 glass adapters – Spider MOD 4752, when using glass adapters MOD 4724 adapters for glass spiders must be used to create off-set away from post.

-           Picket Rail/Vertical bar infill MOD 8926 base and head rail for fixing rod infill. Picket rail is 33.7mm diameter with a 2mm wall thickness and is available in 2500mm lengths. Item can be connected using MOD 5792 easy hit connector. 12mm rod or tube infill available in either 2500mm or 5000mm lengths. Note: All picket fence products are only available in 304 Grade stainless steel

5.6) Accessories: ..........................................

-      MOD 0800 touch up pencil for scratch repair 7016