L30 STAIRS/WALKWAYS/BALUSTRADES

BALUSTRADE(S) AND HANDRAILS(S)

1) Preliminary Remarks

These NBS specifications are intended to simplify the specification process but should be read in conjunction with current building regulations and British standards as well as manufacturers test certificates to ensure the system has full compliance.

We also recommend that you visit our website for CAD drawings, safety certificates, portfolio, news and essential contact details.

Product Guidance:

Our website address: 		www.q-railing.com

Manufacturer:	Q-railing

Product name:  Q-railing, Easy Glass® MOD 0760, MOD 761 or Mod 762 full glass balustrades for fascia fixing of glass, glass handrail profile, glass and accessories.

2) Technical description, Quality characteristics:

-      Fully glazed railing system for face fixing back to substrate

-      Maintenance friendly does not readily corrode, rust or stain.

-      Modular connectors to eliminate the requirement for welding on site.

-      Horizontal uniformly distributed line load, uniformly distributed load applied to infill, and point load to infill testing is available

-      Used separately or fixed to walls using matching handrail brackets

3) Usage:

-      Fittings to be Internal grade (304) or External (316) grades

-      Interchangeable modular components to adapt for raking or horizontal applications, including options for top mounting or fascia fixing of vertical baluster posts

4) Quality/Safety

Even top quality materials are only as good as the workmanship used to process them. Easy installation techniques are created by modular parts with maximum precision fittings. Safety is very important in railing manufacture, vertical and horizontal loadings must be adsorbed. Reference should be made between product test certification and loading requirements set out in the current building regulations and British standards to ensure that the overall system has full compliance.

5.1) Glass adapter side mount for glass railing options:-

-    Glass adapter MOD 0760, 300mm in total height with glass fixings at 230mm centres (angle not adjustable)

-    Glass adapter MOD 0761, 250mm in total height with glass fixing at 150mm centres (angle not adjustable)

-    Glass adapter MOD 0762, 200mm in total height with glass fixing at 150mm centres (angle not adjustable)

5.1.1) Material:

-          Stainless steel Stainless Steel AISI 304

-          Stainless steel Stainless Steel AISI 314

5.1.2) Glass thickness glass adapters

-          MOD 760 for glass thicknesses varying between 12.76mm and 25.52mm

-          MOD 761 for glass thicknesses varying between 10mm and 21.52mm

-          MOD 762 for glass thicknesses varying between 8mm and 17.52mm

Glass thickness: ........................................

Note refer to Q-railing test data for glass specification in relation to specific loading criteria

5.2) Handrail options……………………

-          Diameter 42.4 mm, wall thickness 1.5 mm

-          Diameter 48.3 mm, wall thickness 1.5 mm

-          Rectangular, 60 x 40 mm, wall thickness 1.5 mm

-          Rectangular, 40 x 40 mm, wall thickness 1.5 mm

5.3) Accessories: .................................................................

-          Connectors and end handrails, Stainless Steel AISI 304

-          Connectors and end handrails, Stainless Steel AISI 316

-          Q-glue (Special stainless steel glue)

-          Q-cleaner (stainless steel care spray)

-          Q glass cleaner (glass cleaner)

-          Cementing materials AISI 316

Amount: ................................