L30 STAIRS/WALKWAYS/BALUSTRADES

BALUSTRADE(S) AND HANDRAILS(S)

Detailed description

Balustrade system capable of using varying glass thicknesses from 12mm to 17.52mm toughened and laminated glass.

Top Fixed channel for fixing to top of parapet walls fixings at 200mm centres.

Handrails are available in related stainless steel or wood which can sit directly onto the top of the glass panels using our rubber insert. A number of diameters and profiles are available in different grades. The system is also available with off-set handrails which are fixed through holes in the glass.

The system has been designed to complement the existing range of Q-railing high quality architectural fittings.

We also recommend that you visit our website for CAD drawings, BIM Objects, safety certificates, portfolio, news and essential contact details. It is also recommended that you seek guidance from Q-railing Architectural team to ensure that your specifications are fit for purpose and comply with varying loading requirements.

System includes:

- Aluminium base channel for fixing glass PPC finish RAL 9006 (5000mm long) with various rubber glazing seal options for varying thicknesses of glass

- Choice of handrails including wooden (Beech, Oak lacquered, Beech, P11 stained and lacquered), square, rectangular, circular or elliptical in stainless steel grades 304 or 316 with options for satin or mirror polished surface finish.

- Glass infill available in different thicknesses varying from 12mm to 21.52mm. All glass is bespoke to each specific project this system takes maximum glass height of 800mm.

- Design criteria 0.74kN/m.

Product Guidance:

Even top quality materials are only as good as the workmanship used to process them. Easy installation techniques are created by modular parts with maximum precision fittings. Safety is very important in railing manufacture, vertical and horizontal loadings must be adsorbed. Reference should be made between product test certification and loading requirements set out in the current building regulations and British standards to ensure that the overall system has full compliance.

Product Guidance:

Our website address: 		www.q-railing.com

Product reference: Q-railing, Easy Glass® UP full glass railing system for various mounting applications, for indoor and outdoor use consisting of channel profile, glass handrail profile, glass and accessories.

Base Channel:

Easy Glass UP Top Mount channel

- 16692450036 (5000mm Length) – Stainless Steel effect (RAL9006)

Glazing infill:

Type: As drawing               Suitable glazing thickness: 12mm to 17.52mm

Rubber seals:

- 206600-050-12 for 12mm toughened glass

- 206600-050-14 for 13.52mm toughened laminated glass

- 206600-050-15 for 15mm toughened glass

- 206600-050-18 for 17.52mm toughened laminated glass

Capping rail:

Internal specification

- 136920-042-50-12 – 42.4mm diameter, wall thickness 1.5 mm, 304 Satin 320 Grit

- 136920-048-50-12 – 48.3mm diameter, wall thickness 1.5mm, 304 Satin 320 Grit

- 136924-040-50-12 – 40 x 40mm square, wall thickness 1.5mm, 304 Satin 320 Grit

- 136924-640-25-12 – 60 x 40mm Rectangular, wall thickness 1.5mm, 304 Satin 320 Grit

- 13692004210 – 42.4mm diameter, wall thickness 1.5mm, 304 Mirror polished

- 13692004810 – 48.3mm diameter, wall thickness 1.5mm, 304 Mirror polished

External specification

- 146920-042-50-12 – 42.4mm diameter, wall thickness 1.5 mm, 316 Satin 320 Grit

- 146920-048-50-12 – 48.3mm diameter, wall thickness 1.5mm, 316 Satin 320 Grit

- 146920-060-50-12 – 60.3mm diameter, wall thickness 1.5mm, 316 Satin 320 Grit

- 146924-040-50-12 – 40 x 40mm square, wall thickness 1.5mm, 316 Satin 320 Grit

- 146924-640-25-12 – 60 x 40mm Rectangular, wall thickness 1.5mm, 316 Satin 320 Grit

- 146920-042-50-10 – 42.4mm diameter, wall thickness 1.5mm, 304 Mirror polished

- 14692004810 – 48.3mm diameter, wall thickness 1.5mm, 304 Mirror polished

- 146940-050-14-12 – Edge protection, 14.4mm internal width, 316 Satin 320 Grit

- 146940-050-18-12 – Edge protection, 18.5mm internal width, 316 Satin 320 Grit

- 146940-050-22-12 – Edge protection, 22.6mm internal width, 316 Satin 320 Grit

- 146940-050-26-12 – Edge protection, 26.5mm internal width, 316 Satin 320 Grit

- 166940-050-18-18 – Edge protection, 18.5mm internal width, Brushed Aluminium

- 166940-050-22-18 – Edge protection, 22.5mm internal width, Brushed Aluminium

- 146960-050-24-12 – U profile, internal width 24mm, 316 Satin 320 Grit

- 146360-34 – U profile, internal width 34mm, 316 Satin 320 Grit

- 16694501318 – U Profile, internal width 13.25mm, Brushed Aluminium

- 16694501818 – U Profile, internal width 18mm, Brushed Aluminium

- 166944-050-22-18 – U Profile, internal width 22mm, Brushed Aluminium

- 166924-040-50-18 – 40 x 40mm Square, 24mm internal width, Brushed Aluminium

- 16692004009 – 40 x 40mm Square, 24mm internal width, Aluminium stainless steel effect

- 166924-654-50-08 – 65 x 40mm rectangular profile, Aluminium matt Silver effect

- 166924-654-50-09 – 65 x 40mm rectangular profile, Aluminium Stainless steel effect

- 166924-654-50-00 – 65 x 40mm rectangular profile, Aluminium raw finish

- 166920-042-50-18 – 42.4mm diameter, Brushed Aluminium

- 16692004209 – 42.4mm diameter, Aluminium stainless steel effect

Additional notes

All Cap rails have rubber gaskets to suite extrusion width which varies in accordance with the thickness of the glass. All Cap rails have pre-made accessories including 90 degree corner, adjustable angles, in-line connectors, which are compatible with Q-railing fittings.