L30 STAIRS/WALKWAYS/BALUSTRADES

BALUSTRADE(S) AND HANDRAILS(S)

Detailed description

Balustrade system capable of using varying glass thicknesses from 8mm to 13.52mm toughened and laminated glass.

Slender base fixing with structural support given from either a Top mount post or fascia mount solution. Aluminium Cap rail connects to the head of the balustrade, in either a 33mm x 39mm dimension or 65mm x 40mm.

The system has been designed to complement the existing range of Q-railing high quality architectural fittings.

We also recommend that you visit our website for CAD drawings, BIM Objects, safety certificates, portfolio, news and essential contact details. It is also recommended that you seek guidance from Q-railing Architectural team to ensure that your specifications are fit for purpose and comply with varying loading requirements.

System includes:

- Varying Aluminium base channel for fixing glass (2500mm Long or 5000mm) with various rubber glazing seals for varying glass thicknesses.

- Choice of handrail profiles

- Glass infill available in different thicknesses varying from 8mm to 13.52mm. All glass is bespoke to each specific project.

- Design criteria 0.36, 0.74 and 1.5kN/m.

Product Guidance:

Even top quality materials are only as good as the workmanship used to process them. Easy installation techniques are created by modular parts with maximum precision fittings. Safety is very important in railing manufacture, vertical and horizontal loadings must be adsorbed. Reference should be made between product test certification and loading requirements set out in the current building regulations and British standards to ensure that the overall system has full compliance.

Product Guidance:

Our website address: 		www.q-railing.com

Product reference: Q-railing, Easy Glass® Hybrid full glass railing system for various mounting applications, for indoor and outdoor use consisting of channel profile, glass handrail profile, glass and accessories.

Base Channel:

Easy Glass Hybrid base channel Top Mount channel

- 166910-050-08 (5000mm Length) – Matt silver effect

- 166910-050-00 (5000mm Length) – Raw finish for PPC finish

Glazing infill:

Type: As drawing               Suitable glazing thickness: 8mm to 13.52mm

Rubber seals:

- 206910-08 for 8mm – 8.76mm glass

- 206910-10 for 10mm – 10.76 glass

- 206910-13 for 12.76 – 13.52mm glass

Capping rail:

- 166911-339-50-08 – 33mm W) x 39mm (H) – Square profile (Matt Silver effect)

- 166911-339-50-00 – 33mm (W) x 39mm (H) – Square profile (Raw)

- 166924-654-50-08 – 65mm (W) x 40mm (H) – Rectangular profile (Matt Silver effect)

- 166924-654-50-00 – 65mm (W) x 40mm (H) – Rectangular profile (Raw)

Additional notes

All Cap rails have rubber gaskets to suite extrusion width which varies in accordance with the thickness of the glass. All Cap rails have pre-made accessories including 90 degree corner, adjustable angles, in-line connectors, which are compatible with Q-railing fittings.

Post Options:

Top Mount post

- 11054708034 – Post dimensions are 12mm x 80mm sections. 1040mm High

- 11054808034 – Post dimensions are 12mm x 80mm sections. 1240mm High

Fascia Mount post

- 11054908034 – Post dimensions are 12mm x 80mm sections. 1235mm High

- 11055008034 – Post dimensions are 12mm x 80mm sections. 1435mm High

Additional notes

All handrails have pre-made accessories including 90 degree corner, adjustable angles, in-line connectors, which are compatible with Q-railing fittings.