L30 STAIRS/WALKWAYS/BALUSTRADES

BALUSTRADE(S) AND HANDRAILS(S)

Detailed description

LED Handrail formed from 42.4mm diameter cap rail with strip LED lights fitted within extrusion and associated covering diffusers. The system is designed to be compatible for fixing to walls or as an off-set handrail which are fixed through holes in the glass.

The system has been designed to complement the existing range of Q-railing high quality architectural fittings.

We also recommend that you visit our website for CAD drawings, BIM Objects, safety certificates, portfolio, news and essential contact details. It is also recommended that you seek guidance from Q-railing Architectural team to ensure that your specifications are fit for purpose and comply with varying loading requirements.

Product Guidance:

Even top quality materials are only as good as the workmanship used to process them. Easy installation techniques are created by modular parts with maximum precision fittings. Safety is very important in railing manufacture; vertical and horizontal loadings must be adsorbed. Reference should be made between product test certification and loading requirements set out in the current building regulations and British standards to ensure that the overall system has full compliance.

Product Guidance:

Our website address: 		www.q-railing.com

Product reference: Q-railing, Linear Light® for off-set handrail for wall mounting of fixing to glass

Capping rail:

Internal specification

- 136920-042-50-12 – 42.4mm diameter, wall thickness 1.5 mm, 304 Satin 320 Grit

- 146920-042-50-10 – 42.4mm diameter, wall thickness 1.5mm, 304 Mirror polished

External specification

- 146920-042-50-12 – 42.4mm diameter, wall thickness 1.5 mm, 316 Satin 320 Grit

- 146920-042-50-10 – 42.4mm diameter, wall thickness 1.5mm, 316 Mirror polished

Handrail fixing brackets for various applications:

- 130103-042-12 – Wall mounted handrail bracket with cable duct & 75mm off-set from wall to C/L of handrail, 304 Satin 320 Grit

- 140103-042-12 – Wall mounted handrail bracket with cable duct & 75mm off-set from wall to C/L of handrail, 316 Satin 320 Grit

- 140710-042-01-12 – Post top saddle for 42.4mm diameter post and to accept 42.4mm top rail with cable duct

- 140711-048-01-12 – Post top saddle for 48.3mm diameter post and to accept 48.3mm top rail with cable duct

- 140770-042-12 – Handrail adapter for 42.4mm diameter post and to accept 42.4mm top rail with cable duct

- 14077224812 – Handrail adapter for 48.3mm diameter post and to accept 48.3mm top rail with cable duct

- 140504-042-12 – Wall flange to accept 42.4mm diameter handrail with cable duct

- 140504-048-12 - Wall flange to accept 42.4mm diameter handrail with cable duct

Handrail adapters to accept LED strips and connection to Handrail fixing brackets:

- 140781-24 – Stainless steel connector to allow connection between cap rail and fixing brackets, radiused 21.2mm for 42.4mm diameter fixings

- 140782-24 – Stainless steel connector to allow connection between cap rail and universal adapter, radiused 21.2mm for 42.4mm diameter fixings

- 155092-025 – Plastic insert for clicking in to steel locators as above for mounting LED strip lights. L=2500mm

- 155090-025-24-03 – Diffusers for bottom of cap rail (Clear) – L=2500mm

- 155090-025-24-06 – Diffusers for bottom of cap rail (Frosted) – L=2500mm

LED strips and associated transformers:

Transformers

- 200015-075-20 – 75W IP20 transformer suitable for internal use only

- 19001600000 – 80W IP20 transformer suitable for internal use only

- 200015-100-67 – 80W IP67 transformer suitable for external use

- 19001512000 – 120W IP67 transformer suitable for external use

- 19001700000 – 80W IP67 transformer suitable for external use and also dimmable

LED strips

Indoor

- 41006002700 – K=2700, L=9939mm, Luminous flux per metre = 300 (White)

- 41006006500 – K=6500, L=9939mm, Luminous flux per metre = 340 (White)

- 41006400000 – K=RGB, L=6000mm, Luminous flux per metre = RGB

- 41006102700 – K=2700, L=6000mm, Luminous flux per metre = 600

- 41006106500 – K=6500, L=6000mm, Luminous flux per metre = 700

- 41006500000 – K=RGB, L=4000mm, Luminous flux per metre = 470

Outdoor

- 41006203000 – K=3000, L=6051mm, Luminous flux per metre = 250

- 41006206000 – K=6000, L=6051mm, Luminous flux per metre = 280

- 41003203000 – K=3000, L=6000mm, Luminous flux per metre = 570

- 41003203000 – K=6000, L=6000mm, Luminous flux per metre = 555

Accessories

- 200018 – LED Strip connector (Indoor)

- 200019 – LED strip connection cable (Indoor)

- 19001910000 – LED strip connection cable for LED strip mod 0064 (Indoor)

- 200023 – LED connection sleeve for LED mod 0062 (Outdoor)

- 19001601000 – Remote control & cable for mod 0016.020 (RGB – Indoor)

- 200012-001-01 – L=100m, Black cable 1mm2

- 200012-001-02 – L=100m, Red cable 1mm2

Additional notes

All handrails have pre-made accessories including 90 degree corner, adjustable angles, in-line connectors, which are compatible with Q-railing fittings.