12-136097L30 STAIRS/WALKWAYS/BALUSTRADES

BALUSTRADE(S) AND HANDRAILS(S)

Detailed description

Balustrade system capable of using varying glass thicknesses from 19mm to 31.52mm toughened and laminated glass.

Top Fixed, in either standard top mounted or F profile options. Fascia mounted in either standard Fascia mounted profile or Y Channel. (Hole centres dependent on structure and required loading performance.

Handrails are available in related stainless steel or wood which can sit directly onto the top of the glass panels using our rubber insert. A number of diameters and profiles are available in different grades. The system is also available with off-set handrails which are fixed through holes in the glass.

The system has been designed to complement the existing range of Q-railing high quality architectural fittings.

We also recommend that you visit our website for CAD drawings, BIM Objects, safety certificates, portfolio, news and essential contact details. It is also recommended that you seek guidance from Q-railing Architectural team to ensure that your specifications are fit for purpose and comply with varying loading requirements.

System includes:

- Varying Aluminium base channel for fixing glass (2500mm Long) with various rubber glazing seal options for fixed or adjustable applications.

- Choice of handrails including square, rectangular, circular or elliptical in stainless steel grades 304 or 316 with options for satin or mirror polished surface finish.

- Glass infill available in different thicknesses varying from 19mm to 31.52mm. All glass is bespoke to each specific project.

- Design criteria 1.5kN/m to 3kN/m based on specific test situations

Product Guidance:

Even top quality materials are only as good as the workmanship used to process them. Easy installation techniques are created by modular parts with maximum precision fittings. Safety is very important in railing manufacture, vertical and horizontal loadings must be adsorbed. Reference should be made between product test certification and loading requirements set out in the current building regulations and British standards to ensure that the overall system has full compliance.

Product Guidance:

Our website address: 		www.q-railing.com

Name:		Q-railing

Telephone:	01782 740410

Fax:		0800 781 4246

Email:		ukconsultant@q-railing.com

Address:	Q-railing UK

Unit 1, Tunstall Arrow

James Brindley Way, Stoke-on-Trent

Staffordshire, ST6 5GF

Product reference: Q-railing, Easy Glass® Max full glass railing system for various mounting applications, for indoor and outdoor use consisting of channel profile, glass handrail profile, glass and accessories.

Base Channel:

Easy Glass Max Top Mount channel

- 16.6926.000.18 (2500mm Length) – Stainless Steel effect

Easy Glass Max Fascia Mounted channel

- 16.6935.000.18 (2500mm Length) – Stainless Steel effect

Profile available with either:-

- 24.6915.030.36 ABS Blind bungs for covering fixing holes

- 16.6920.518.18 Aluminium stainless steel effect cladding

Easy Glass Max F Channel

- 16.6929.000.18 (2500mm Length) – Stainless Steel effect

Easy Glass Pro Y Channel

- 16.6936.000.18 (2500mm Length) – Stainless Steel effect

Profile available with Cladding to the front side product reference 16.6920.520.18

Glazing infill:

Type: As drawing               Suitable glazing thickness: 19mm to 31.52mm

Rubber seals:

Fixed wedge set (No adjustment)

- 19.6907.919.00 for 19mm toughened glass

- 19.6907.922.00 for 21.52mm toughened glass

- 19.6907.925.00 for 25mm toughened glass

- 19.6907.926.00 for 25.52mm toughened laminated glass

- 19.6901.932.00 for 31.52mm toughened laminated glass

Note refer to Q-railing test data for glass specification in relation to specific loading criteria required for various building types. Email ukconsultant@q-railing.com

Capping rail:

Internal specification

- 13.6920.042.12 – 42.4mm diameter, wall thickness 1.5 mm, 304 Satin 320 Grit

- 13.6920.048.12 – 48.3mm diameter, wall thickness 1.5mm, 304 Satin 320 Grit

- 13.6920.060.12 – 60.3mm diameter, wall thickness 1.5mm, 304 Satin 320 Grit

- 13.6920.040.12 – 40 x 40mm square, wall thickness 1.5mm, 304 Satin 320 Grit

- 13.6925.640.12 – 60 x 40mm Rectangular, wall thickness 1.5mm, 304 Satin 320 Grit

External specification

- 14.6920.042.12 – 42.4mm diameter, wall thickness 1.5 mm, 316 Satin 320 Grit

- 14.6920.048.12 – 48.3mm diameter, wall thickness 1.5mm, 316 Satin 320 Grit

- 14.6920.060.12 – 60.3mm diameter, wall thickness 1.5mm, 316 Satin 320 Grit

- 14.6920.040.12 – 40 x 40mm square, wall thickness 1.5mm, 316 Satin 320 Grit

- 14.6925.640.12 – 60 x 40mm Rectangular, wall thickness 1.5mm, 316 Satin 320 Grit

- 14.6920.042.10 – 42.4mm diameter, wall thickness 1.5mm, 304 Mirror polished

- 14.6920.048.10 – 48.3mm diameter, wall thickness 1.5mm, 304 Mirror polished

- 20.6945.118.44 – Edge protection, 18.5mm internal width, Oak glazed

- 20.6945.122.44 – Edge protection, 22.5mm internal width, Oak glazed

- 14.6940.114.12 – Edge protection, 14.4mm internal width, 316 Satin 320 Grit

- 14.6940.118.12 – Edge protection, 18.5mm internal width, 316 Satin 320 Grit

- 14.6940.122.12 – Edge protection, 22.6mm internal width, 316 Satin 320 Grit

- 14.6940.126.12 – Edge protection, 26.5mm internal width, 316 Satin 320 Grit

- 16.6940.118.18 – Edge protection, 18.5mm internal width, Brushed Aluminium

- 16.6940.122.18 – Edge protection, 22.5mm internal width, Brushed Aluminium

- 14.6940.030.12 – U profile, internal width 24mm, 316 Satin 320 Grit

- 14.6340.040.12 – U profile, internal width 34mm, 316 Satin 320 Grit

- 16.6945.013.18 – U Profile, internal width 13.25mm, Brushed Aluminium

- 16.6945.018.18 – U Profile, internal width 18mm, Aluminium Brushed Aluminium

- 16.6940.022.18 – U Profile, internal width 22mm, Aluminium Brushed Aluminium

- 16.6920.040.18 – 40 x 40mm Square, 24mm internal width, Brushed Aluminium

- 16.6920.654.08 – 65 x 40mm rectangular profile, Aluminium matt Silver effect

- 16.6920.654.00 – 65 x 40mm rectangular profile, Aluminium raw finish

- 16.6920.042.18 – 42.4mm diameter, Aluminium stainless steel effect

Additional notes

All Cap rails have rubber gaskets to suite extrusion width which varies in accordance with the thickness of the glass. All Cap rails have pre-made accessories including 90 degree corner, adjustable angles, in-line connectors, which are compatible with Q-railing fittings.

Off-set handrail option:

Glass to Glass handrail brackets:

Internal Specification

- 13.0148.000.12 – Handrail bracket for square rail fixing – 77mm of set to C/L of handrail, 304 Satin

- 13.0148.044.12 – Handrail bracket for either 42.4 or 48.3mm – 77mm to C/L of handrail, 304 Satin

- 13.0117.000.12 – Handrail bracket for square rail fixing – 77mm of set to C/L of handrail, 304 Satin

- 13.0117.042.12 – Handrail bracket for 42.4mm diameter, 77 to C/L of handrail, 304 Satin

- 13.0117.048.12 – Handrail bracket for 48.3mm diameter, 77 to C/L of handrail, 304 Satin

- 13.9350.042.12 – Handrail bracket for 42.4mm diameter, 107 to C/L of handrail, 304 Satin

- 13.9350.048.12 – Handrail bracket for 48.3mm diameter, 107 to C/L of handrail, 304 Satin

External Specification

- 14.0148.000.12 – Handrail bracket for square rail fixing – 77mm of set to C/L of handrail, 316 Satin

- 14.0148.044.12 – Handrail bracket for either 42.4 or 48.3mm – 77mm to C/L of handrail, 316 Satin

- 14.0117.000.12 – Handrail bracket for square rail fixing – 77mm of set to C/L of handrail, 316 Satin

- 14.0117.042.12 – Handrail bracket for 42.4mm diameter, 77 to C/L of handrail, 316 Satin

- 14.0117.048.12 – Handrail bracket for 48.3mm diameter, 77 to C/L of handrail, 316 Satin

- 14.9350.042.12 – Handrail bracket for 42.4mm diameter, 107 to C/L of handrail, 316 Satin

- 14.9350.048.12 – Handrail bracket for 48.3mm diameter, 107 to C/L of handrail, 316 Satin

Handrail Options:

Internal Specification

- 13.0900.242.12 – 42.4mm diameter, wall thickness 1.5mm, 304 Satin 320 Grit

- 13.0900.248.12 – 48.3mm diameter, wall thickness 1.5mm, 304 Satin 320 Grit

- 13.4900.041.12 – 40 x 10mm rectangular, wall thickness 1.5mm, 304 Satin 320 Grit

- 13.4900.040.12 – 40 x 40mm Square, wall thickness 1.5mm, 304 satin 320 Grit

External Specification

- 14.0900.242.12 – 42.4mm diameter, wall thickness 1.5mm, 316 Satin 320 Grit

- 14.0900.248.12 – 48.3mm diameter, wall thickness 1.5mm, 316 Satin 320 Grit

- 14.4900.041.12 – 40 x 10mm rectangular, wall thickness 1.5mm, 316 Satin 320 Grit

- 14.4900.040.12 – 40 x 40mm Square, wall thickness 1.5mm, 316 satin 320 Grit

Additional notes

All handrails have pre-made accessories including 90 degree corner, adjustable angles, in-line connectors, which are compatible with Q-railing fittings.

For UK specification queries please use the above contact information to liaise directly with Q-railings specification team to