L30 STAIRS/WALKWAYS/BALUSTRADES

BALUSTRADE(S) AND HANDRAILS(S)

1) Preliminary Remarks

These NBS specifications are intended to simplify the specification process but should be read in conjunction with current building regulations and British standards as well as manufacturers test certificates to ensure the system has full compliance.

We also recommend that you visit our website for CAD drawings, safety certificates, portfolio, news and essential contact details.

Product Guidance:

Our website address: 		www.q-railing.com

Company:	Q-railing

Telephone:	0800 781 4245

Fax:		0800 781 4246

Email:		ukconsultant@q-railing.com

Address:	Q-railing UK

Unit 1. Tunstall Arrow

James Brindley way

Stoke-on-Trent

Staffs

ST6 5GF

Manufacturer:	Q-railing

Product name:  Square Line®, modular stainless steel balustrade system

2) Technical description, Quality characteristics:

-      Square profiled modular balustrading system

-      Maintenance friendly does not readily corrode, rust or stain.

-      Modular connectors to eliminate the requirement for welding on site.

-      Horizontal uniformly distributed line load, uniformly distributed load applied to infill,     and point load to infill testing is available

-      Various base fitting options

-      Used separately or fixed to walls using matching handrail brackets

3) Usage:

-      Internal (304) or External (316) grades

-      Interchangeable modular components to adapt for raking or horizontal applications, including options for top mounting or fascia fixing of vertical baluster posts

4) Quality/Safety

Even top quality materials are only as good as the workmanship used to process them. Easy installation techniques are created by modular parts with maximum precision fittings. Safety is very important in railing manufacture, vertical and horizontal loadings must be adsorbed. Reference should be made between product test certification and loading requirements set out in the current building regulations and British standards to ensure that the overall system has full compliance.

5.1) Railing profile: brushed, square tube measuring 40 x 40mm with 2mm wall thickness

5.1.1) Material:

Stainless Steel AISI 304

Stainless Steel AISI 316

5.1.2) Mass: ...........................................

5.1.3) Surface treatment K 320 ground

5.2) Post, square: 40 x 40 mm, thickness 2.0 mm

5.3) Floor flanges / railing post mounts

-     Floor flanges model 4942 with cover cap model 4511 (Adjustable for raking applications is model 4942)

-     Baluster bracket options model 4551, 4555, 4556, 4558, 4550, 0550

5.4) Handrail: .........................................

Square 40 x 40 mm, wall thickness 2.0 mm model reference 4900, 4925 with connectors reference 4301, 4304, 4790, 4312, 4317, 4318.

5.5 Infill: ..........................................

-      Rod, 15 x 5 mm

-      Glass clamp, glass, thickness: ...........mm

-      Note: glass Terminal EN 12600-03 tested ACC. DIN.

-      Glass thickness from 6 to 15 mm and 6.76 to 17.76 mm

-      Plate adapter

5.6) Handrail wall: ...................

-      Square 40 x 40 mm, thickness 2.0 mm

-      Tubes 40 x 10 mm, Wall thickness 1.5 mm

5.7) Accessories: ..........................................

-      Handrail support for mounting to wall

-      Handrail support for fastening on tube

-      Handrail support for mounting on glass

-      Connectors and end handrails, stainless steel Stainless Steel AISI 304

-      Connectors and end handrails, stainless steel Stainless Steel AISI 316

-      Cross Rod holder

-      Adhesives (Special stainless steel glue)

-      Cementing materials in stainless steel AISI 316

For UK specification queries please use the above contact information to liaise directly with Q-railings specification team to ensure that the full specification is received correctly.