L30 STAIRS/WALKWAYS/BALUSTRADES

BALUSTRADE(S) AND HANDRAILS(S)

1) Preliminary Remarks

These NBS specifications are intended to simplify the specification process but should be read in conjunction with current building regulations and British standards as well as manufacturers test certificates to ensure the system has full compliance.

We also recommend that you visit our website for CAD drawings, safety certificates, portfolio, news and essential contact details.

Our website address: 	www.q-railing.com

Company:	Q-railing

Telephone:	0800 781 4245

Fax:		0800 781 4246

Email:		ukconsultant@q-railing.com

Address:	Q-railing UK

Unit 1, Tunstall Arrow

James Brindley Way, Stoke-on-Trent

Staffordshire, ST6 5GF

Manufacturer:	Q-railing

Product name:  Square Line®60x30, modular stainless steel balustrade system

2) Technical description, Quality characteristics:

-      Rectangular profiled modular balustrading system

-      Maintenance friendly does not readily corrode, rust or stain.

-      Horizontal uniformly distributed line load, uniformly distributed load applied to infill, and point load to infill testing is available

-      Various base fitting options

3) Usage:

-      External (316) grades

-      Interchangeable modular components to adapt for raking or horizontal applications, including options for top mounting or fascia fixing of vertical baluster posts

4) Quality/Safety

Even top quality materials are only as good as the workmanship used to install them. Easy installation techniques are created by modular parts with maximum precision fittings. Safety is very important in railing manufacture, vertical and horizontal loadings must be adsorbed. Reference should be made between product test certification and loading requirements set out in the current building regulations and British standards to ensure that the overall system has full compliance.

5.1) Railing profile: brushed, square tube measuring 60 x 30mm with 2.6mm wall thickness

5.1.1) Material:

Stainless Steel AISI 316

5.1.2) Mass: ...........................................

5.1.3) Surface treatment K 320 ground

5.2) Baluster Posts

- Square: 60 x 30 mm, wall thickness 2.6 mm MOD 4901 – including welded base flange MOD 3503. Height of post = 1000mm

- Square: 60 x 30mm, wall thickness 2.6mm MOD 4902 – Height of post = 1250mm

5.3) Floor flanges/railing post mounts

-     Floor flanges model 3503 measuring 110mm x 110mm with 4 No. 14mm dia. Holes for bolting      down to substrate.  Cover cap model 4513 measuring 115mm x 115mm square. The base flange comes as standard with the MOD 4901 post or separate for bespoke post sizes.

-     Baluster bracket option for side mounting of the posts MOD 4558

5.4.1) Handrail fixing methods:

Handrail to be connected to the top of the posts using modular handrail brackets:-

-       MOD 4711 for mounting of square handrail.

-       MOD 4711 for mounting of 42.4mm or 48.3mm diameter tubular handrail

-       MOD 4725 for mounting of variable handrails to accommodate raking applications and corner applications. Relevant models for reference purposes are MOD 0810, MOD 0818, MOD 0811, MOD 0812, MOD 1800, MOD 1801, MOD 1802.

5.4.2) Handrail options

The standard handrail for the system is MOD 4900 measuring 60 x 30 x 2.6mm L=5000mm

However the handrail fixing methods accommodate all various Q-railing handrail options from 42.4, 48.3 to 40 x 40mm

5.5 Infill

-      Glass clamps most appropriate are MOD 42 (Others are also available), glass thickness: varies between 6 and 12.76mm. Glass thickness varies in accordance with loading requirements

Note: glass Terminal EN 12600-03 tested ACC. DIN.

-      Glass holders, MOD 55 for glass thickness: varies between 8 – 13.52mm.

-      Glass Adaptors MOD 4752 & MOD 4753: varies between 8 – 13.52mm

-      Plate Adapters MOD2800 For plate infills 2,3,4mm

-      Picket Rail/Vertical bar infill MOD 8926 base and head rail for fixing rod infill. Picket rail is 33.7mm diameter with a 2mm wall thickness and is available in 2500mm lengths. Item can be connected using MOD 5792 easy hit connectors. 12mm rod or tube infill available in either 2500mm or 5000mm lengths. Note: All picket fence products are only available in 304 Grade stainless steel

5.7) Accessories:

-      Connector for varying angles 0 - 70˚ MOD 4312

-      Connector for straight connections MOD 5792

-      500mm x 500mm 90˚ corner section purpose made for corners MOD 4320

-      Easy Hit end caps MOD 5727

-      Cementing materials in stainless steel AISI 316

For UK specification queries please use the above contact information to liaise directly with Q-railings specification team to ensure that the full specification is received correctly.